Instruction Manual

Hydraulic Drum Dumpers

DA45                    DA45-1

Note:The Owner/Operator must read carefully and understand all the information presented here before operation

DA45 series Drum dumper Figure

DA45                               DA45-1

DA45                            DA45-1

Preface

l. In order to make the users quickly understand the structure and performance features of this product, so as to carry out regular maintenance as well as correct regulation of vehicle, our factory hereby compile this operation instruction sheet as well as spare parts table for users' reference.

2. Read the instruction sheet and parts table carefully before using and make clear the structural performance, Maintenance as well as attentions.

3. The personnel without license cannot operate the drum dumper. Only those qualified personnel with professional training as well as approved by relevant departments and familiar with the structural performance can independently operate the vehicle.

4. The work site of drum dumper shall be flat, solid place with enough loading capacity.

5. Our company will not hold any responsibility for the modification of drum rotator by users without the prior approval of our company.

Table of Contents

1、Brief introduction

2、Main technical performance and parameter

3、Technical Data

4、Safety operation

5、Maintenance

1、Brief  Introduction

Safe drum dumping requires well-designed, ergonomic drum handling equipment to reduce risks of injury or property damage. hydraulic Drum Karriers, drum dumpers DA45 combine incredible strength with simple drum handling by one person. It can help you lift, move and pour plastic, fiber and steel drums, and also can provide controlled dispensing of your drums.

With the feature of simple design and easy operation, this all steel construction Hydraulic Drum Dumper is what you need to dump steel & plastic drums that weigh up to 450kg.

2.Main Technical Performance and Parameter

2.1

2.2. Feature Of

Hydraulic Drum Dumpers

1. Floor lock;

2. Load capacity 450kg;

3. Base leg span 640mm;

4. Combined Eagle-grip structure and Anchorear structure together;

3. Suitable for handling steel and plastic drums;

4. Can be operated by one people

6. Manually rotate drums 180 degree;

7. Can keep drums at any angle;

8. The solid steel construction provides stability during transit;

9. Lowers drums to the floor;

10. All steel construction;

11. Drum Karriers provide controlled dispensing of your drum;

12. Suitable for 55 gallon steel drums & 210L rimmed plastic drums;

13. Widely used for chemical plants and food plants for transferring, loading & unloading, charging and compounding materials of fluids, such as dyes, paintings, adhesives, petrol, edible oils, etc

14. Special rolling technology manufacturing "C" type door frame, strong security

3、Technical Data

| Model |  | Hydraulic Drum Dumpers, Drum Handler DA45 |
| --- | --- | --- |
| Load capacity | kg | 450 |
| Net weight | Kg | 176 |
| Lifting height | mm | 1500 |
| Pouring height | mm | 1580 |
| Caster dimension | mm | Ø150*50 |
| Inner/Outer width of front support legs | mm | 650/810 |
| Max. Grade ability (Fully-loaded/no-load) | % | 3/5 |
| Overall size | mm | 1400*1000*2100 |

4、Safety operation

Operation of electric drum dumper

- Open the anchor ear, adjust the lock ring-pull, making the anchor ear hold tightly the drum which need be transported. The plastic drum should use limiting stopper.

- It can customize pedal and manually type , lifting the drum to the specified height. It can manually  rotate 360 degree.

5. Maintenance

Weekly inspection

1. Check and screw up bolts and nuts;

2. Add butter to the UPC bearing every week regularly;

3. Change oil every a half year;

| NO. | Name | Unit | Qty | NO. | Name | Unit | Qty |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Scroll wheel | pcs | 4 | A19 | Drum clasp | pcs | 1 |
| A2 | 205 Bearing | pcs | 4 | A20 | M8X20 sunk screw | pcs | 2 |
| A3 | φ52 Hole Stop Collar | pcs | 4 | A21 | M8 nut | pcs | 2 |
| A4 | Inner Tower | pcs | 1 | A22 | Bearing pedestal | pcs | 2 |
| A5 | Clamp | pcs | 1 | A23 | M10X30 bolt | pcs | 4 |
| A6 | Fixed pin | pcs | 1 | A24 | φ10 Flat Gasket | pcs | 4 |
| A7 | R type pin | pcs | 1 | A25 | Speed Reducer | pcs | 1 |
| A8 | Clamp frame | pcs | 1 | A26 | M10X40 bolt | pcs | 4 |
| A9 | Hoop 1 | pcs | 1 | A27 | M10 Nut | pcs | 4 |
| A10 | Hoop 2 | pcs | 1 | A28 | Pin 1 | pcs | 1 |
| A11 | Hoop 3 | pcs | 1 | A29 | Connecting rod | pcs | 1 |
| A12 | Hoop Roller | pcs | 2 | A30 | 205 Bearing | pcs | 1 |
| A13 | Hoop Pin | pcs | 2 | A31 | Hand wheel | pcs | 1 |
| A14 | Fastening Pieces | pcs | 1 | A32 | Pin 2 | pcs | 1 |
| A15 | M8X15 bolt | pcs | 2 | A33 | Gasket | pcs | 1 |
| A16 | M8 Nut | pcs | 6 | A34 | M5X10 bolt | pcs | 1 |
| A17 | Fastening chain | pcs | 1 | A35 | M12X80 bolt | pcs | 4 |
| A18 | Spring | pcs | 1 | A36 | M12 Nut | pcs | 4 |

| No. | Description | Qty |
| --- | --- | --- |
| 1 | valave basement | 1 |
| 2 | valve | 1 |
| 3 | lever plate | 1 |
| 4 | big spring 5.5x48x88 | 1 |
| 5 | spring hat 48x15 | 1 |
| 6 | valve core 18x95 | 1 |
| 7 | copper bushing 2015 | 4 |
| 8 | axle 20x106 | 1 |
| 9 | elastic pin 8x30 | 1 |
| 10 | wheel basement | 1 |
| 11 | lever pin 16 | 1 |
| 12 | piston 31.5x910 | 1 |
| 13 | nut assy | 1 |
| 14 | spring 2.5x18x188 | 1 |
| 15 | rubber protection 32x91 | 1 |
| 16 | circlip 12x1 | 2 |
| 17 | shaft | 1 |
| 18 | shaft axle | 1 |
| 19 | frame assy | 1 |